# ANNEX II + III:	 TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

# Contract title: Supply of Agricultural machines 3	p 1 /…

Publication reference: 3/IOT/2022-Agricultural Machine 3

Columns 1-2 should be completed by the contracting authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the contracting authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words ‘compliant’ or ‘yes’ are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Lot 1: Tractors 1

Tractor with Engine power in the range 130-140 HP  …………………………………………………………..  3

Lot 2: Tractors 2

Tractor with Engine power in the range 100-115 HP  ………………………………………………………….. 7

| 1. Item number | 2. Specifications required | 3. Specifications offered | 4. Notes, remarks, ref to documentation | 5. Evaluation committee’s notes |  |
| --- | --- | --- | --- | --- | --- |
| Lot 1: Tractors 1 |  |  |  |  |
| 1.1 | Tractor with Engine power in the range 130-140 HP Type: Open field tractor, with 4 wheels drive (4WD). With heavy duty water separator filter for poor fuel. Radiator for hot and dry climates. Tire2/ STAGEII or STAGEIIIA/Tier3, without SCR (Selective Catalysis Reduction) technology. Engine: Four strokes Diesel engine, 4 or 6 cylinders in line, direct injection or common rail, turbo/air to air intercooler, water cooled. Max. engine power: in the range 130-140 HP, without considering the boosting power system Max. torque: Not less than 500 Nm Transmission: Full power shift or semi power shift with creeper gear. Brake: Wet discs in oil bath and independent hand brake |  |  |  |  |
|  | Hydraulic system: total main pump flow standard (litre/min): not less 80 litre/min Hydraulic remote valves: REAR: not less 4 double acting. Hitch: REAR: 3 points linkage cat II and III. Max. lift capacity at ball ends not less than 7 ton. Hydraulic top linkage with quick attach hooks. control switches on rear mudguards for: (1) hitch and (2) PTO Steering: Hydrostatic Weight without additional ballast: Not less than 5.0 tons Ballast: Front ballast of 400 kg and rear ballast of 200 kg for balancing of tractor and equipment connected. PTO: REAR: not less than two speeds at 540/1000 RPM CAB: ROPS enclosed Cabin (Roll Over Protective Structures) or FOPS enclosed cabin (Falling Object Protective Structures) opening windows (right and left), air conditioned, and pneumatic suspension seat with seat belt. Cabin fire extinguisher. Tires: Points reference (specify in case of difference tyres): REAR TYRES: not less than 540/65R38. FRONT TYRES: correspondent. |  |  |  |  |
|  | Consumables/Spare parts/accessories (For each of the machines)*: 20 engine oil filters, 5 hydraulic oil filters, 5 air engine filters, 20 fuel filters, 5 air conditioning filters, 10 water filters, additional kit bolts / pins for fixing front and rear wheels. Specific (proprietary) tools set shall be provided per each machine – machinery jack per each tractor not less 10 tons, fire extinguisher, hand greasing nipples not less than 25 kg. One for each tractor: PTO shaft, water pump, hydraulic pumps, full set of belts, turbo charger. For the lot: one complete of enging overhaul. Warranty and after sale services: - Warranty up to 2 years - On-site services upon request, including repairs and spare parts where necessary for up to 12 interventions in the 2-year warranty. Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. Training (minimum requirements) **: Training on-site for drivers, technicians and mechanics after the delivering of the machines (diagnosis and usage of the tractor). Requirements for trainers: 3 trainers per site, for 3 days per site |  |  |  |  |
| *Consumables/Spare Parts/accessories: The above list provided the minimum requirements, and each item should be quoted in the financial offer. Additional consumables/spare parts/accessories should be added to reach 10% of the value of the total contract Price. Quantities of each additional item can be offered by tenderers on the basis of their professional experience and the expected places of use. **Trainings: The training description provides the minimum requirements. An enhanced training package can be offered by tenderers on the basis of their Professional experience and the expected places of use. Note: At least 10 calendar days before the delivering of the training, the contractor must provide to the Contracting Authority a detailed agenda of training along with the contents of modules for their approval. |  |  |  |  |  |
| Lot 2: Tractors 2 |  |  |  |  |  |
| 2.1 | Tractor with power in the range 100-115 HP Type: Open field tractor, with 4 wheels drive. With heavy duty water separator filter for poor fuel. Radiator for hot and dry climates. STAGEIIIA/Tier3, without SCR (Selective Catalysis Reduction) technology. Engine: Four strokes Diesel engine, 4 or 6 cylinders in line, direct injection, turbo/air to air intercooler, water cooled. Max. engine power: in the range 100-115 HP without considering boosting power system Max. torque: not less than 400 Nm Transmission: Semi power shift or mechanical synchronized, number of the speeds not less than 12FWD – 12 REV. Creeper. Lever for forward/reversible direction. |  |  |  |  |
|  | Brake: Wet discs in oil bath and independent hand brake Hydraulic system: total main pumps flow not less 50 litre/min Hydraulic remote valves: REAR: not less 2 double acting. Hitch: REAR: 3 points linkage cat II / III. Max. lift capacity at ball ends not less than 4.4 ton. Steering: Hydrostatic Weight without additional ballast (ton): Not less than 4.0 tons Ballast: Front ballast of 400 kg and rear ballast of 200 kg for balancing of tractor and equipment connected. PTO: REAR: not less than two speeds at 540/1000 RPM CAB: ROPS enclosed Cabin (Roll Over Protective Structures) or FOPS enclosed cabin (Falling Object Protective Structures) opening windows (right and left), air conditioned, and pneumatic suspension seat with seat belt. Cabin fire extinguisher |  |  |  |  |
|  | Tires: Points reference (specify in case of difference tyres): REAR TYRES: 480/70 R34 or more. FRONT TYRES: correspondent. Consumables/Spare parts/accessories (For each of the machines)*: 20 engine oil filters, 5 hydraulic oil filters, 5 air engine filters, 20 fuel filters, 5 air conditioning filters, 10 water filters, additional kit bolts / pins for fixing front and rear wheels. Specific (proprietary) tools set shall be provided per each machine – machinery jack per each tractor not less 10 tons, fire extinguisher, hand greasing nipples not less than 25 kg. One for each tractor: PTO shaft, water pump, hydraulic pumps, full set of belts, turbo charger. For the lot: two complete of enging overhaul. |  |  |  |  |
|  | Warranty and after sale services: - Warranty up to 2 years - On-site services upon request, including repairs where necessary for up to 12 interventions in the 2-year warranty. Manual: English (N.2 copies or CD) and preferable Arabic language shall be provided per each machine including complete manual of of maintenance, spare parts and assemble and disassemble instructions Training (minimum requirements) ** for all the items of Lot 3: Training on-site for drivers, technicians and mechanics after the delivering of the machines (diagnosis and usage of the tractor). Numbers of trainer/day: 3 trainers per site, for 3 days per site |  |  |  |  |
| *Consumables/Spare Parts/accessories: The above list provided the minimum requirements, and each item should be quoted in the financial offer. Additional consumables/spare parts/accessories should be added to reach 10% of the value of the total contract Price. Quantities of each additional item can be offered by tenderers on the basis of their professional experience and the expected places of use. **Trainings: The training description provides the minimum requirements. An enhanced training package can be offered by tenderers on the basis of their Professional experience and the expected places of use. Note: At least 10 calendar days before the delivering of the training, the contractor must provide to the Contracting Authority a detailed agenda of training along with the contents of modules for their approval. |  |  |  |  |  |