ANNEX II + III:  TECHNICAL SPECIFICATIONS + TECHNICAL OFFER

Contract title: Supply of Agricultural machines 2 	p 1 /43

Publication reference: 2/IOT/2019-Agricultural Machine 2

Columns 1-2 should be completed by the Contracting Authority

Columns 3-4 should be completed by the tenderer

Column 5 is reserved for the evaluation committee

Annex III - the Contractor's technical offer

The tenderers are requested to complete the template on the next pages:

- Column 2 is completed by the Contracting Authority shows the required specifications (not to be modified by the tenderer),

- Column 3 is to be filled in by the tenderer and must detail what is offered (for example the words “compliant” or “yes” are not sufficient)

- Column 4 allows the tenderer to make comments on its proposed supply and to make eventual references to the documentation

The eventual documentation supplied should clearly indicate (highlight, mark) the models offered and the options included, if any, so that the evaluators can see the exact configuration. Offers that do not permit to identify precisely the models and the specifications may be rejected by the evaluation committee.

The offer must be clear enough to allow the evaluators to make an easy comparison between the requested specifications and the offered specifications.

Lot 1: Tractors 1	4

Tractor with Engine power not less than 195 HP	4

Lot 2: Tractors 2	7

Tractor with Engine power in the range 130-140 HP	7

Lot 3: Tractors 3	10

Tractor with power in the range 100-115 HP	10

Lot 4: Tractors 4	14

Tractor with power in the range 80- 90 HP	14

Lot 5: Combine	19

SELF-PROPELLED combine harvester	19

Lot 6: Agriculture machines and implements 1	23

Complete laser unit	23

Disc harrow 36 discs X shape	25

Potato planter 4 rows	27

Potato haulm topper 2 rows (rear mounted)	28

Potato digger 2 rows	29

Sugar beet haulm topper 6 rows	31

Sugar beet digger 6 rows	32

Sugar beet loader	33

Excavator trailed by tractor	34

Lot 7: Agriculture machinery and implements-2	36

Chisel subsoiler (Ripper)	36

Reaper bar for tractor	37

Two wheels tractor with equipment	38

Pneumatic Planter 6 rows with fertilizer distributor	39

Sugarbeet digger 3 row	40

Lot 8: Logistics Vehicles	42

Workshop tools kit	42

| 1. Item Number | 2. Specifications Required (Minimum Technical Specifications Required): | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| Lot 1: Tractors 1 |  |  |  |  |
| 1.1 | Tractor with Engine power not less than 195 HP Type: Open field tractor, with 4 wheels drive (4WD). Without any SCR (Selective Catalysis Reduction) technology. STAGEIIIA/Tier3. With heavy duty water separator filter for fuel. Radiator f8or hot and dry climates. Engine: Four strokes Diesel engine 6 cylinders in line, direct injection or common rail, turbo/air to air intercooler, water cooled. Max engine power (HP): Not less than 195 HP Max. torque (Nm): Not less than 800 Nm. Transmission: Full power shift or semi power shift with creeper gear. Brake: Wet discs in oil bath and independent handbrake Hydraulic system: total main pump flow standard (litre/min): Not less 85 litre/min Hydraulic remote valves: REAR: not less 4 double acting. Hitch: REAR: 3 points linkage cat II & III. Max. Lift capacity at ball ends not less than 9 ton. Hydraulic cylinder for height adjustment of the right arm of the hitch. Hydraulic top linkages with quick attach hooks. Control switches on rear mudguards. Steering: Hydrostatic Weight without additional ballast (ton): Not less than 6.5 tons Ballast: Front weight with adequate mass for optimum balancing of tractor and equipment connected (not less than 800 kg). PTO: REAR: not less than two speeds at 540/1000 RPM, PTO shaft at 6 splines and PTO shaft at 21 splines. CAB: ROPS (Roll Over Protective Structures) enclosed Cabin or FOPS (Falling Object Protective Structures) enclosed Cabin with opening windows, air conditioned, suspension seat with seat belt. Fire extinguisher for cabin. Tires: Points reference (specify in case of difference tyres): REAR TYRES 650/65R42. Correspondent FRONT TYRES. Consumables (For each of the machine)*: 20 engine oil filters, 5 hydraulic oil filters, 10 air engine filters, 20 fuel filters, 5 air conditioning filters, 10 water filters, additional kit bolts / pins for fixing front and rear wheels, specific (proprietary) tools set shall be provided per each machine – hydro- jack per each tractor not less 10 tons, fire extinguisher, hand greasing nipples not less than 25 kg. Spare PTO shaft (6/21- splines) Diagnostic hardware and software (For each of the machine): N° 1 Diagnostic tools, with not less 11 inches display; waterproof technology; designed for heavy duty use in vehicle repair workshops. With N. 1 Software for diagnosis for present tractors. (These are not necessary in case already provided for present tractors either in previous category or other lots) Warranty and after sale services: - Warranty up to 2 years - All included 4 on-site services for a maximum of 2 years. Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. --------------------------- Training (minimum requirements) **: - Training on-site for drivers, technicians and mechanics after the delivering of the machines (diagnosis and usage of the tractor). Requirements for trainers: 2 trainers per site, for 2 days per site |  |  |  |

*Consumables/Spare parts: The list provided the minimum requirements. Additional consumables/spare parts/accessories can be offered by tenderers on the basis of their Professional experience and the expected places of use.

**Trainings: The training description provides the minimum requirements. An enhanced training package can be offered by tenderers on the basis of their Professional experience and the expected places of use.

Note: At least 10 calendar days before the delivering of the training, the contractor must provide to the Contracting Authority a detailed agenda of training along with the contents of modules for their approval.

| 1. Item Number | 2. Specifications Required (Minimum Technical Specifications Required): | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| Lot 2: Tractors 2 |  |  |  |  |
| 2.1 | Tractor with Engine power in the range 130-140 HP Type: Open field tractor, with 4 wheels drive (4WD). With heavy duty water separator filter for poor fuel. Radiator for hot and dry climates. Tire2/ STAGEII or STAGEIIIA/Tier3, without SCR (Selective Catalysis Reduction) technology. Engine: Four strokes Diesel engine, 4 or 6 cylinders in line, direct injection or common rail, turbo/air to air intercooler, water cooled. Max. engine power: in the range 130-140 HP, without considering the boosting power system Max. torque: Not less than 500 Nm Transmission: Full power shift or semi power shift with creeper gear Brake: Wet discs in oil bath and independent hand brake Hydraulic system: total main pump flow standard (litre/min): not less 80 litre/min Hydraulic remote valves: REAR: not less 4 double acting. Hitch: REAR: 3 points linkage cat II and III. Max. lift capacity at ball ends not less than 7 ton. Hydraulic top linkage with quick attach hooks. control switches on rear mudguards for: (1) hitch and (2) PTO Steering: Hydrostatic Weight without additional ballast: Not less than 5.0 tons Ballast: Front and rear ballast standard with adequate mass for optimum balancing of tractor and equipment connected. PTO: REAR: not less than two speeds at 540/1000 RPM CAB: ROPS enclosed Cabin (Roll Over Protective Structures) or FOPS enclosed cabin (Falling Object Protective Structures) opening windows (right and left), air conditioned, and pneumatic suspension seat with seat belt. Cabin fire extinguisher. Tires: Points reference (specify in case of difference tyres): REAR TYRES: not less than 540/65R38. FRONT TYRES: correspondent. Additional tires : N. 4 narrow tyres and rim with reference width of about 270 -300 mm for each tractor Consumables (For each of the machines)*: 20 engine oil filters, 5 hydraulic oil filters, 10 air engine filters, 20 fuel filters, 5 air conditioning filters, 10 water filters, additional kit bolts / pins for fixing front and rear wheels. Specific (proprietary) tools set shall be provided per each machine – machinery jack per each tractor not less 10 tons, fire extinguisher, hand greasing nipples not less than 25 kg, PTO shaft for each tractor. Diagnostic hardware and software: N° 1 Diagnostic tools each delivery site, with not less 11 inches display; waterproof technology; designed for heavy duty use in vehicle repair workshops. With N. 1 Software for diagnosis for present tractors. (These are not necessary in case already provided for present tractors either in previous category or other lots) Warranty and after sale services: - Warranty up to 2 years - All included 4 on-site services for a maximum of 2 years. Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. Training (minimum requirements) **: - Training on-site for drivers, technicians and mechanics after the delivering of the machines (diagnosis and usage of the tractor). Requirements for trainers: 2 trainers per site, for 2 days per site |  |  |  |
| Lot 3: Tractors 3 |  |  |  |  |
| 3.1 | Tractor with power in the range 100-115 HP Type: Open field tractor, with 4 wheels drive. With heavy duty water separator filter for poor fuel. Radiator for hot and dry climates. STAGEIIIA/Tier3, without SCR (Selective Catalysis Reduction) technology. Engine: Four strokes Diesel engine, 4 or 6 cylinders in line, direct injection, turbo/air to air intercooler, water cooled. Max. engine power: in the range 100-115 HP without considering boosting power system Max. torque: not less than 400 Nm Transmission: Semi power shift or mechanical synchronized, number of the speeds not less than 12FWD – 12 REV. Creeper. Lever for forward/reversible direction. Brake: Wet discs in oil bath and independent hand brake Hydraulic system: total main pumps flow not less 50 litre/min Hydraulic remote valves: REAR: not less 2 double acting. Hitch: REAR: 3 points linkage cat II / III. Max. lift capacity at ball ends not less than 4.4 ton. Steering: Hydrostatic Weight without additional ballast (ton): Not less than 4.0 tons Ballast: Front and rear ballast standard with adequate mass for optimum balancing of tractor and equipment connected. PTO: REAR: not less than two speeds at 540/1000 RPM CAB: ROPS enclosed Cabin (Roll Over Protective Structures) or FOPS enclosed cabin (Falling Object Protective Structures) opening windows (right and left), air conditioned, and pneumatic suspension seat with seat belt. Cabin fire extinguisher Tires: Points reference (specify in case of difference tyres): REAR TYRES: 480/70 R34 or more. FRONT TYRES: correspondent. Additional tires: N. 4 narrow tyres and rim with reference width of 270-280 mm (REAR TYRES) per each tractor. Consumables (For each of the machines) *: 20 engine oil filters, 5 hydraulic oil filters, 10 air engine filters, 20 fuel filters, 5 air conditioning filters, 10 water filters, additional kit bolts / pins for fixing front and rear wheels. Specific (proprietary) tools set shall be provided per each machine – machinery jack per each tractor not less 5 tons, fire extinguisher, hand greasing nipples not less than 15 kg, PTO shaft for each tractor. Diagnostic hardware and software (For each of the machine): N° 1 Diagnostic tools, with not less 11 inches display; waterproof technology; designed for heavy duty use in vehicle repair workshops. With N. 1 Software for diagnosis for present tractors. (These are not necessary in case already provided for present tractors either in previous category or other lots) Warranty and after sale services: - Warranty up to 2 years from the date of the provisional acceptance - At least all included 4 on-site services for a maximum of 2 years from the date of the provisional acceptance by authorised companies. Manual: English (N.2 copies or CD) and preferable Arabic language shall be provided per each machine including complete manual of of maintenance, spare parts and assemble and disassemble instructions. Training (minimum requirements) ** for all the items of Lot 3: Training on-site for drivers, technicians and mechanics after the delivering of the machines (diagnosis and usage of the tractor). Numbers of trainer/day: 2 trainers per site, for 2 days per site |  |  |  |
| Lot 4: Tractors 4 |  |  |  |  |
| 4.1 | Tractor with power in the range 80- 90 HP Type: tractor, with 4 wheels drive (wheels with different diameter between the front and rear axle). With heavy duty water separator filter for fuel. Radiator for hot and dry climates. STAGE II/Tier2 or STAGEIIIA/Tier3, without SCR (Selective Catalysis Reduction) technology. Engine: Four strokes Diesel engine, 4 cylinders in line, direct injection, turbo/air to air intercooler, water cooled. Max. engine power: in the range 80 -90HP, without boosting power system Max. torque: Not less than 250 Nm Transmission: Semi power shift or mechanical synchronized, number of the speeds not less 12FWD – 12 REV. (Preferable with creeper if it is available on the offered model) Brake: Wet discs in oil bath and independent hand brake Hydraulic system: total main pumps flow not less 40 litre/min Hydraulic remote valves: REAR: not less 2 double acting. Hitch: REAR: 3 points linkage and lift capacity at ball ends not less than 3.0 ton. Steering: Hydrostatic Wheelbase: 1900 – 2100 mm Ground Clearance: 240-290 mm Weight without additional ballast (ton): Not less than 2.5 tons Ballast: Front and rear ballast standard with adequate mass for optimum balancing of tractor and equipment connected. PTO: REAR: not less than two speeds at 540/1000 RPM Rollover protection structure: folding type Tires: Points reference (specify in case of difference tyres): REAR TYRES: 420/70 R24 or more. FRONT TYRES: correspondent. Consumables (For each of the machines) *: 20 engine oil filters, 5 hydraulic oil filters, 10 air engine filters, 20 fuel filters, 5 air conditioning filters, 10 water filters, additional kit bolts / pins for fixing front and rear wheels. Specific (proprietary) tools set shall be provided per each machine – machinery jack per each tractor not less 5 tons, fire extinguisher, hand greasing nipples not less than 25 kg, PTO shaft for each tractor. Diagnostic hardware and software (For each of the machine): N° 1 Diagnostic tools with not less 11 inches display; waterproof technology; designed for heavy duty use in vehicle repair workshops. With N. 1 Software for diagnosis for present tractors. (These are not necessary in case already provided for present tractors either in previous category or other lots) Warranty and after sale services: - Warranty up to 2 years from the date of the provisional acceptance - At least all included 4 on-site services for a maximum of 2 years from the date of the provisional acceptance by authorised companies. Manual: English (N.2 copies or CD) and preferable Arabic language shall be provided per each machine including complete manual of maintenance, spare parts and assemble and disassemble instructions. Training (minimum requirements) ** : Training on-site for drivers, technicians and mechanics after the delivering of the machines (diagnosis and usage of the tractor). Numbers of trainer/day: 2 trainers per site, for 2 days per site |  |  |  |

*Consumables/Spare parts: The list provided the minimum requirements. Additional consumables/spare parts/accessories can be offered by tenderers on the basis of their Professional experience and the expected places of use.

**Trainings: The training description provides the minimum requirements. An enhanced training package can be offered by tenderers on the basis of their Professional experience and the expected places of use.

Note: At least 10 calendar days before the delivering of the training, the contractor must provide to the Contracting Authority a detailed agenda of training along with the contents of modules for their approval.

| 1. Item Number | 2. Specifications Required (Minimum Technical Specifications Required): | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| Lot 5: Combine |  |  |  |  |
| 5.1 | SELF-PROPELLED combine harvester Type: Transverse flow and 4 wheels drive. With heavy duty water separator filter for fuel. STAGEIIIA/Tier3, without SCR (Selective Catalysis Reduction) technology. Crops: Wheat, barley, maize Engine: Four stroke Diesel engine, direct injection or common rail. Max. engine power: not less 230 HP Fuel tank capacity: Not less than 400 litres Transmission: Hydrostatic, with not less 3 gears/ 4 speeds. Tires: Point of reference (specify in case of different tyres): REAR: 460/70R24. FRONT: 650/75R32 Brake: Hydraulically or mechanical independent Wheat-barley head: Not less than 6m width Maize head: Number of rows 8 and row distance 0.7 m - 0.8 m Cutting height: Can be controlled from driver cabin with auto float control Crop elevator: Not less 3 chains Drum width: Not less 1.3 m Separating system: Straw-walkers’ separation surface not less 5 m2 Fan: Volumetric high efficiency with speeds control from the cabin Wheat tank capacity: Not less than 7000 litres Wheat handling system: Through unloading auger hydraulically operated in all directions. Cabin: Air conditioned with all indicators. The cabin must be equipped with a right-hand armrest with joystick for the easy control of the machine’s operations. Cabin fire extinguisher. Electric circuit: Complete electric circuit with day and night lighting system, dynamo and starter motor heavy duty type Accessories: - 1 trailer for transport of wheat-barley header on public road - 1 trailer for transport of maize header on public road Consumables (For each of the machines) *: 20 engine oil filters, 5 transmission oil filters, 10 air engine filters, 20 fuel filters, 5 air conditioning filters, 10 water filters, additional kit bolts / pins for fixing front and rear wheels, consumables for the header (50 tines + 50 knife). Specific (proprietary) tools set shall be provided per each machine – machinery jack per each tractor not less 10 tons, fire extinguisher, hand greasing nipples not less than 25 kg. Diagnostic hardware and software (For each of the machine): N° 1 Diagnostic tool, with not less 11 inches display; waterproof technology; designed for heavy duty use in vehicle repair workshops. With N. 1 Software for diagnosis. (These are not necessary in case already provided for present tractors either in previous category or other lots) Warranty and after sale services: - Warranty up to 2 years - All included 4 on-site services for a maximum of 2 years. Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. Training (minimum requirements) **: - Training on-site for drivers, technicians and mechanics after the delivering of the machines (diagnosis and usage of the tractor). Requirements for trainers: 2 trainers per site, for 2 days per site |  |  |  |

*Consumables/Spare parts: The list provided the minimum requirements. Additional consumables/spare parts/accessories can be offered by tenderers on the basis of their Professional experience and the expected places of use.

**Trainings: The training description provides the minimum requirements. An enhanced training package can be offered by tenderers on the basis of their Professional experience and the expected places of use.

Note: At least 10 calendar days before the delivering of the training, the contractor must provide to the Contracting Authority a detailed agenda of training along with the contents of modules for their approval.

| 1. Item Number | 2. Specifications Required (Minimum Technical Specifications Required): | 3. Specifications Offered | 4. Notes, remarks, ref to documentation | 5. Evaluation Committee’s notes |
| --- | --- | --- | --- | --- |
| Lot 6: Agriculture machines and implements 1 to be operated with tractor of lots 1 and 2 |  |  |  |  |
| 6.1 | Complete laser unit Laser transmitter Battery: 2 batteries rechargeable and easy to dismantle and installation on the site, The number of working hours per one battery not less than 45 h, with a total number of operating hours of 90 h Battery Charger: Works on 220 volts Suit case: Carrying case of shock resistant type Component: A long wooden tripod with extended legs made of aluminium with height 2.7 m. The fixed base of the device is equipped with a middle mast for raising and lowering with length of a 0.6 m with a quick coupling joint Laser receiver 360 ° - Works with 9 channels with a cable connected from the unit to the control box operating range at least 200 m, with carrying case and connecting cable. Electric mast: telescopic electric mast with the length of the fixed part 1.5 m and movable part about 0.8 m operate on 12 or 24 volts Control Box: • connect and control all the laser equipment on the tractor and blade • on/off switch • rise/lower both mast and blade switch • equipped with protection fuse Set of Operating Cables: Battery cable - Receiver cable to control box - solenoid cable From control box to hydraulic unit Electric hydraulic valve unit: The system works on closed and open systems and other systems on all types of tractors with different HP and can withstand pressure up to 280 bars and made of steel Installation set: Includes all the bases, cables, connectors and hoses required for operation Consumables: suggested by tenderer Manual: English (N.2 copies or CD) and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 6.2 | Disc harrow 36 discs X shape General description: Disc harrow equipped with 4 groups of discs mounted in X shaped Linkage: Towing eye drawbar Working width: Not less 4.0 m Moving on the road: Hydraulic folding frame transport on the road. The transporting width of the folded disc harrow must not exceed 2.7 m. The frame must have equipped with not less 2 rubber tires to match the disc harrow weight when moving on the roads. Settings: Hydraulically device to adjust the tilt angles of the discs with the direction of motion Group of discs: Each group equipped with 9 discs and each set must be fixed on at least 2 bearings base with a suitable size and heavy duty, and to be equipped with a grease nipple or in oil bath. The two front sets fitted with notched discs and the two rear sets fitted with a smooth disc. Discs: The discs should be made of temperate steel, diameter not less than 600 mm and thickness not less than 6.0 mm. Scrapers: Each disc provided with a suitable shape scraper with the possibility to control the clearance between the disc and the scraper The roller: Space rollers between discs made of cast iron Chassis: The discs are mounted on a suitable heavy-duty chassis Cage roller on the back: YES Lighting and warning board kit: YES Needed Power of the tractor: lower than 190 HP Consumables (minimum requirements) *: n. 16 notched discs and N. 16 smooth disc. Manual: English (N.2 copies or CD) and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 6.3 | Potato planter 4 rows Number of the rows: 4 Seeding capacity: not less 1 ha/h Hopper: equipped with a hydraulic system to lower and raise the hopper and facilitate the loading operation. Capacity not less than 1500 kg Sowing distances between rows: from 75 to 90cm Sowing on the row: 15-40cm Consumables: suggested by tenderer Warranty and after sale services: 2 years Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 6.4 | Potato haulm topper 2 rows (rear mounted) Type: mounted to the 3 points back hitch of the tractor Number of rows: 2 rows Operation width: not less than 1.8 m Hitching: 3 points linkage cat II & III Source of power: PTO shaft at a speed of 540/1000 RPM Gearbox: In the middle of the machine with, heavy duty gears operated at an high power tractors and a set of clutches to protect gear box. Means of transmitting the motion from gear box to the drum: By means of a side belts with a movable tensioner that is easily controlled and must indicate the number of belts Wheels (Rubber): The machine is equipped with a rubber wheel (common size) and equipped with a means to adjust the height of the machine from the ground Knives: Thermally treated steel and fastened to the rotor shaft in a simple way and in a different shape. Cover: Equipped with a suitable device to shape the row. Cardanic shaft: With suitable double single joint can allow joint angles (12-13°) for each joint. The cardanic shaft must be incorporated with an “Adjustable Overrunning Clutch Friction Torque Limiters”. Consumables (minimum requirements) *: A set of supplementary knives is to be delivered with the machine Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 6.5 | Potato digger 2 rows Type: Potato digger 2 row trailed behind tractor or mounted to the 3 points back hitch of the tractor Number of rows: 2 rows One row width: 0.9 m Operating width: not less than 1.8 m Digging knives: The machine is equipped with separate front knives for each row of tubers (6-8 knives). It is equipped with a means that convey the tubers to the web, equipped also with 4 disks as (front guide) and is equipped with 2 metal rollers Source of power: PTO shaft at a speed of 540 or 1000 RPM through a flexible telescopic universal joint with clutch and circlip for hanging on when storing. Transport mat: The machine must be is equipped with a single web to convey the crop with length not less than 7 m and a width not less than 1.5 m in one stage and equipped with vibrating totally made of molded rubber or with a lock and bars galvanized or covered with rubber to prevent crop damaging. Cardanic shaft: With suitable double single joint can allow joint angles (12- 13°) for each joint. The cardanic shaft must be incorporated with an “Adjustable Overrunning Clutch Friction Torque Limiters”. Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 6.6 | Sugar beet haulm topper 6 rows Type: Sugar beet topper rear mounted on the tractor Number of rows: 6 rows Spacing between rows: Suitable for working of 0.45 m spacing with easy adjustment PTO required: 540 or 1000 RPM Cutting height adjustment: Cutting height adjustment is achieved by means a hydraulic cylinder directly connected to the tractor hydraulic circuit and two support wheels on the machine. Leaf's scatter system: Spread on the soil. Wheels (Rubber): The machine should be equipped with rubber wheels (common size) Cardanic shaft: With suitable double single joint can allow joint angles not less (12-13°) for each joint. The cardanic shaft must be incorporated with an “Adjustable Overrunning Clutch Friction Torque Limiters”. Consumables*: A set of supplementary cutting tools is to be delivered with the machine. Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 6.7 | Sugar beet digger 6 rows Type: mounted to the 3 points back hitch of the tractor Number of rows: 6 rows Spacing between rows: Suitable for working of 0.45 m spacing with easy adjustment 3pt. Hitch required: Cat. II – III PTO required: 540 or 1000 RPM Digger system: Vibrating motion of the ploughshares Digger height adjustment: Digger height adjustment is achieved by means a hydraulic system and two support wheels on the machine Swathing beet: YES Cardanic shaft: With suitable double single joint can allow joint angles (12- 13°) for each joint. The cardanic shaft must be incorporated with an “Adjustable Overrunning Clutch Friction Torque Limiters”. Consumables: suggested by tenderer Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 6.8 | Sugar beet loader Type: Sugar beet loader trailed of the tractor for harvesting the beets being in the swath Linkage: Towing eye drawbar Harvesting and cleaning system: Belt made of top-quality hardened steel and soil-removing rollers Storage box capacity: Not less than 12 tons full loaded Box emptying system: By means of a mechanical or hydraulic system that allows discharge inside a trailer PTO required: 540 or 1000 RPM Lighting and warning board kit: YES Cardanic shaft: With suitable double single joint can allow joint angles (12-13°) for each joint. The cardanic shaft must be incorporated with an “Adjustable Overrunning Clutch Friction Torque Limiters”. Consumables: suggested by tenderer Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 6.9 | Excavator trailed by tractor Weight of excavator (with the 2 wheels and without ballast): not less than 1700 kg; Width of excavator: from 2.0 - 2.5 meter; Frame length: from 2.5 to 3 meter; Raising capacity: at full arm with attachment on not less than 750 kg; at half arm with attachment not less than 1300 kg; Rotation: constant rotation in both directions 360° Oil tank capacity: not less than 40 l; Tires: n.2 standard tires (references point: 11-5-15); Hydraulic unit and distribution set: not less than 50 l With fork or grab bucket (both to be included): working height: no less than: 3.70 meter; working distance: not less than 5.00 meter; working depth: not less than 3.25 meter; With reversed dredging shovel (to be included): working height: not less than: 5.50 meter; working distance: not less than 5.5 meter; working depth: not less than 3.00 meter; - The excavator should have a comfort seat for the operator and rear lights; - The excavator should include all the accessories and necessary attachment cables to operate with the tractor; - Excavator should be operated with tractor lot n.2 Consumables: suggested by tenderer. Provide list of consumables and spare parts for complete overall maintenance should provided Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions |  |  |  |
|  | Training (minimum requirements) ** for all the items of Lot 6: Training on-site for drivers (of the related tractor), technicians and mechanics after the delivering of the machines (use and maintenance, linkage to tractor, safety). Numbers of trainer/day: 2 trainers per site, for 2 days per sit |  |  |  |
| Lot 7: Agriculture machinery and implements-2 to be operated with tractor of lots 3 and 4 |  |  |  |  |
| 7.1 | Chisel subsoiler (Ripper) Type: mounted on the tractor, 3-point hitch of cat. II or III Working width: between 2.6 to 2.8 m Shanks: number:7 shanks with pull bolt safety device Frame height: not less 0.5 m Weight: at least 600 kg Accessories: N.1 twin spike rollers with the linkage systems Consumables: suggested by tenderer Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 7.2 | Reaper binder Type: self propelled reaper binder for the cutting and binding sheaves of wheat, sesame and other grain crops. The reaper binder must be 4 wheels and must be equipped with a seat for transporting the driver. Engine: Four strokes Diesel engine Max. engine power: not less than 10 Hp Transmission: Mechanical synchronized, number of the speeds not les 3 FWD – 1 REV. Cutter width: not less than 1.3m Consumables : 30 rolls of twine for tying, 30 cutting units of the cutter bar and 30 alignment fingers of the cutter bar Warranty and after sale services: - Warranty up to 2 years Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 7.3 | Reaper bar for tractor Type: Double blade sickle bar mower mounted on the rear side of the tractor Working width: not less than 1.4 m Mowing bar lifting system: by means of hydraulic system PTO required: 540/1000 RPM Cardanic shaft: with suitable double single joint can allow joint angles not less 25° for each joint. Consumables: suggested by tenderer Manual: English (N.2 copies or CD) and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 7.4 | Two wheels tractor with equipment Engine: Four strokes Diesel engine Max. engine power: not less than 10 Hp Transmission: Mechanical synchronized, number of the speeds not les 3 FWD. PTO: independent from gearbox Weight (without attachment): not less than 120 kg Tires: Points reference (specify in case of difference tyres): 6.5/80-12 Equipment: N.1 rotary hoe with working width not less than 0.8 m; N. 1 ridger, N1 Mower Consumables: suggested by tenderer but including spares for at least 4 services Warranty and after sale services: - Warranty up to 2 years Manual: English and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 7.5 | Pneumatic Planter 6 rows with fertilizer distributor Type: mounted on the rear of the tractor Type of frame: fixed Type of distribution system: pneumatic, also for fertilizer Number of rows: N.6 - with inter rows distance 0.45 m interchangeable to N.4 - with inter rows distance 0.75 m Tank for seeds: not less than 30 L per unit Fertilizer: not less 500 L Sowing disc: 1 complete set of sowing discs for corn, cotton, beet, sunflowers and peanuts Clods pusher: yes Furrow opener: coulter Seed covered: double blade Hydraulic row marker: yes Cardanic shaft: with suitable double single joint can allow joint angles not less 25° for each joint. Manual: English (N.2 copies or CD) and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
| 7.6 | Sugarbeet digger 3 row Type: mounted on the tractor, 3-point hitch Number of rows: 3 rows Spacing between rows: Suitable for working of 0.45 m spacing with easy adjustment PTO required: 540 or 1000 RPM Digger system: Vibrating motion of the ploughshares Digger height adjustment: by means mechanical or hydraulic system Swathing beet: YES Cardanic shaft: With suitable double single joint can allow joint angles not less 25° for each joint. The cardanic shaft must be incorporated with an “Adjustable Overrunning Clutch Friction Torque Limiters”. Consumables: suggested by tenderer Manual: English (N.2 copies or CD) and preferable Arabic language shall be provided per each machine including complete manual of maintenance and spare parts instructions. |  |  |  |
|  | Training (minimum requirements) ** for all the items of Lot 7: Training on-site for drivers, technicians and mechanics after the delivering of the machines (diagnosis and usage of the self-propelled machines, use and maintnenance, linkage to tractor, safety of operators). Numbers of trainer/day: 2 trainers per site, for 2 days per site |  |  |  |
| Lot 8: Logistics Vehicles |  |  |  |  |
| 8.1 | Workshop tools kit Consisting in n.1 portable tools chest made in steel, heavy-duty, suitable for mechanics with lockable drawers: 1 top drawers, at least 3 large drawers, small drawers and 1 big cabinet on the bottom, a handle on one side and four swivel casters The tool-boxes shall be fully equipped with at least the followings and any further necessary to regulate and provide maintenance of all machines and implements requested in present tender (lots 1,2,3,4,5,6,7): - Screw drivers set (7+7) - Iron hammers (1.0, 1.5, 2.0 kg) - Rubber hummer - Driver female set and socket wrench 6-36 mm (24 pieces) - Wrench set (16-36 mm) - Wrench set stencil ENG (6, 8, 10 inch) - Adjustable wrench (8, 10, 14, 18 inch) - Torque key 0.5 inch (40-210 N) - Torque key 0.5 inch (70-390 N) - Filter key - Allen key complete set - Adjustable filler tools (4 pieces) - Needle nose pliers (4 pieces – 2 sets) - Clamp pliers (2 pieces) - Mini needle nose plies (10 inch) - Volt-ohm meter device - Digital micrometre (0-25<,25-90,90-75 mm) - Electricity cable led - Box level (40 cm) - Calliper (6-8-10 inch) - Wire cutter (6 inch) - Flat chisel set - Reaming set of 3:20 mm - Hand saw - File set (8 inch- 5 pieces) |  |  |  |